FIDELITY CHESS COMPUTER

# VERSION 11

Technical Data

Author: Wilfried Bucke						Date: December 07, 2003

The hardware has been developed some years ago in order to increase the performance of old Fidelity chess computer models like Elite (one display), Elite Avant Garde (two displays) and Prestige. It was adapted to the Motorola processor MC68060 with max. 72 MHz, but can also be powered by the MC68040 processor as it is used in the original version 10. When upgrading the chess board no external changes are to be done.

Technical details:

Processor:	MC68EC060RC75 (75 MHz Version)

Processor clock:	72 MHz

Bus clock:	36 MHz

Program size:	4x 256Kx8  EPROM   (1 MB)

Cache:	4x     8Kx8  SRAM      (32 KB)

Memory size:	4x 512Kx8  SRAM       (2 MB Hash Tables)

Permanent Memory:	2x     8Kx8  EEPROM (16 KB, non volatile)

CPU Fan:	yes

Power consumption:	approx. 10 Watt

Because of the heat emission the device must be operated on a smooth surface only (no tablecloth!) and should be swiched on and off by the AC line only.

Software:

- The version 11 program is completely identical with the version 10 program.

- Due to the display windows with only 4 digits it is not possible to correctly show higher  values of nodes per second (activated by Square H2) calculated by the 68060 processor.

Test program (press TB during the start routine):

- If the hardware is installed in a Fidelity chess computer with one display window only like Elite or Prestige, a different hardware jumper setting is necessary. In this case an error in the test program will occur!

- The test program is not able to display a memory size with more than 1024 KB.

The test program does not have any influence on the program of the chess computer !!

Delivery volume:

- New hardware, including installation and testing

- 1 power supply ( 5V DC, 2.5 Ampere),  including  power cord

- 1 user manual (in English)

- 1 name plate to be sticked